Technical Review Package

Austin Residential Plan Check  ·  HOME Phase I  ·  2406 Euclid Avenue

| Project | 2406 Euclid Avenue, Austin TX 78722 |
| --- | --- |
| Review Date | 2026-03-01 |
| Governing Phase | PHASE_I — HOME Phase I, Ord. 20231207-001 |
| Use Type | TWO UNIT RESIDENTIAL |
| Sheets Reviewed | 32 |
| State Preemption | None triggered |

## Key Issues

- Lot area inconsistency (7,330 vs 7,730 sq ft) — affects all FAR calculations

- ADU GFA not submitted — cannot confirm per-unit or site FAR compliance

- ADU lacks street-facing entrance toward Euclid Ave

- Use type must be formally designated as Two-Unit Residential (§25-2-901 repealed)

## At a Glance

| PASS | FAIL | UNCLEAR | Corrections | [VERIFY] |
| --- | --- | --- | --- | --- |
| 11 | 5 | 8 | 5 | 7 |

## Findings by Code Basis

| Code Basis | Findings | Confidence Breakdown |
| --- | --- | --- |
| HOME Phase I: 9 Subchapter F: 2 LDC Base District: 2 IRC 2021: 2 IBC 2021: 1 TX State: 1 | Total: 23 Pass: 11 Fail: 5 Unclear: 8 | HIGH: 7 MEDIUM: 11 LOW: 5 |

# Lot Classification

Source: lot_classification.json

| Lot Area (permit app) | 7330 sq ft |
| --- | --- |
| Lot Area (site table) | 7730 sq ft ⚠ DISCREPANCY |
| Lot Area Used | 7330 sq ft (lower value, conservative) |
| Lot Width (est.) | ~55 ft (approximate, from site plan visual read — exact dimension not confirmed) |
| Flag Lot | false |
| Zoning | SF-3-NP — Dawson Neighborhood Plan |
| Units Proposed | 2 |
| Use Type | TWO UNIT RESIDENTIAL |
| Governing Phase | PHASE_I |
| Governing Section | Austin LDC §25-2-773, HOME Phase I, Ord. 20231207-001 (eff. Dec 18 2023; applies to apps filed on/after Feb 5 2024) |
| Phase II Applicable | No — Phase II (§25-2-779, Ord. 20240516-006) governs lots between 1,800–5,749 sq ft. This lot exceeds 5,750 sq ft |
| Subchapter F | Yes — FAR limits of §25-2-773(E) apply |
| Confidence | MEDIUM |

## Unit Descriptions

| Unit | Description |
| --- | --- |
| Unit 1 | 2-story house w/ habitable attic, fronting Euclid Ave (north) |
| Unit 2 | 2-story ADU w/ attached single-car garage, at rear of lot (south, adjacent to alley) |

Lot Area Discrepancy Note:  Permit application (page 4) states 7,330 sq ft; site development data table (pages 5-6) states 7,730 sq ft. Difference: 400 sq ft. Both values exceed the 5,750 sq ft Phase I threshold — governing phase is PHASE_I regardless. Discrepancy must be resolved by licensed surveyor and corrected in all submittal documents.

# Sheet Manifest

Source: sheet-manifest.json  ·  32 pages  ·  Stamp date: 10.25.2023

| Pg | Sheet ID | Description |
| --- | --- | --- |
| 1 | BSPA-1 | Austin Energy BSPA Application Form |
| 2 | BSPA-2 | Austin Energy BSPA / Owner Authorization |
| 3 | AUTH | Owner Authorization / AE forms continuation |
| 4 | APP | City of Austin Permit Application |
| 5 | SDE-1 | Site Development Info — project data table |
| 6 | SDE-2 | Site Development Info — building coverage / impervious cover calculations |
| 7 | SubF-1 | Subchapter F GFA Calculation Form — House w/ Habitable Attic (page 1) |
| 8 | SubF-2 | Subchapter F GFA Calculation Form — House w/ Habitable Attic (page 2) |
| 9 | p.1a | Site Plan — overall site with setbacks, coverage, impervious |
| 10 | p.1a-2 | Site Plan — continuation / enlarged view |
| 11 | p.1b | Tree / Demolition Plan |
| 12 | p.1c | Impervious Cover Exhibit |
| 13 | p.2a1 | First Floor Plan — House |
| 14 | 6.105 | Tree Protection Standard Details (6.105-1 through 6.105-4) |
| 15 | p.1d | Proposed Site Plan — AE Review version |
| 16 | UNKNOWN-16 | Sheet not re-read in current session — see prior session notes |
| 17 | p.2a2 | Upper (Second) Floor Plan — House |
| 18 | p.2a3 | Habitable Attic Floor Plan + Sections A-A, B-B, C-C + Attic Area Calculations — House |
| 19 | p.2b | Ground Floor Plan + Upper Floor Plan — ADU (with attached garage) |
| 20 | UNKNOWN-20 | Sheet not re-read in current session — see prior session notes |
| 21 | p.3e1 | East Elevation + North Elevation (street-facing) + Setback Tent Diagram — House |
| 22 | p.3a2 | West Elevation + South Elevation + Roof Plan — House |
| 23 | p.3e1-ADU | South Elevation + East Elevation + Sections AA & BB + Setback Tent Diagram — ADU |
| 24 | p.3b2 | North Elevation + West Elevation — ADU |
| 25 | UNKNOWN-25 | Sheet not re-read in current session — see prior session notes |
| 26 | S200 | Structural Foundation Details — Greensmith Engineering, signed PE, dated 10/27/23 |
| 27 | S300 | 2nd Floor Framing Plans — House and ADU |
| 28 | S301 | Attic Floor / Roof Framing Plans — House and ADU |
| 29 | S302 | House Roof Framing Plan |
| 30 | UNKNOWN-30 | Sheet not re-read in current session — see prior session notes |
| 31 | S304 | 2nd Floor Wall Bracing Plans — House and ADU |
| 32 | S400 | Structural General Notes + Wall Section Details + Typical Wall Bracing |

# Full Findings — All 23 Checks

Source: sheet_findings.json

## Administrative

| ID | Check | Status | Conf | Sheet | Page |
| --- | --- | --- | --- | --- | --- |
| A-01 | Lot area — consistency across submittal documents | FAIL | HIGH | APP, SDE-1 | 4, 5 |
| A-02 | Use type designation — Two-Unit Residential vs. SFR+ADU | FAIL | HIGH | APP, SubF-1 | 4, 7 |
| A-03 | HOME Phase I ordinance number — cited in applicable codes list | UNCLEAR | MEDIUM | APP, SDE-1 | 4, 5 |

## Site

| ID | Check | Status | Conf | Sheet | Page |
| --- | --- | --- | --- | --- | --- |
| A-04 | Building coverage — Phase I maximum 40% | PASS | HIGH | SDE-2, p.1a | 6, 9 |
| A-05 | Impervious cover — Phase I maximum 45% | PASS | HIGH | SDE-2, p.1c | 6, 12 |
| A-06 | Front yard impervious cover — Phase I maximum 40% | UNCLEAR | LOW | p.1a, p.1c | 9, 12 |
| A-07 | Front setback — house (primary building) | UNCLEAR | MEDIUM | p.1a, p.3e1 | 9, 21 |
| A-08 | Rear setback — ADU (adjacent to alley) | UNCLEAR | MEDIUM | p.1a, p.2b | 9, 19 |
| A-09 | Side setbacks — both buildings | UNCLEAR | MEDIUM | p.1a, p.2b | 9, 19 |
| A-10 | Front porch setback — house | UNCLEAR | MEDIUM | p.1a, p.3e1 | 9, 21 |
| A-11 | Parking — front/street yard spaces | UNCLEAR | MEDIUM | p.1a, p.1d | 9, 15 |

## Architectural

| ID | Check | Status | Conf | Sheet | Page |
| --- | --- | --- | --- | --- | --- |
| B-01 | House per-unit GFA — Subchapter F FAR limit (Two-Unit Residential) | UNCLEAR | MEDIUM | SubF-1, SubF-2, p.2a1, p.2a2, p.2a3 | 7, 8, 13, 17, 18 |
| B-02 | ADU per-unit GFA — Subchapter F FAR limit not shown | FAIL | HIGH | SubF-1, p.2b | 7, 19 |
| B-03 | Combined site GFA — Subchapter F site FAR limit (Two-Unit Residential) | FAIL | HIGH | SubF-1, SubF-2, p.2b | 7, 8, 19 |
| B-04 | House height — base district maximum 35 ft | PASS | HIGH | p.3a2 | 22 |
| B-05 | ADU height — base district maximum 35 ft | PASS | HIGH | p.3e1-ADU | 23 |
| B-06 | Street-facing entrance — House (Unit 1) | PASS | HIGH | p.3e1 | 21 |
| B-07 | Street-facing entrance — ADU (Unit 2) | FAIL | MEDIUM | p.3b2, p.3e1-ADU, p.2b | 24, 23, 19 |
| B-08 | Habitable attic GFA calculation methodology — House | PASS | HIGH | p.2a3 | 18 |
| B-09 | Egress windows — all sleeping rooms | PASS | MEDIUM | p.2a2, p.2b | 17, 19 |
| B-10 | Garage placement — ADU (parking structure setback from front lot line) | PASS | MEDIUM | p.2b, p.3e1-ADU | 19, 23 |

## Life Safety

| ID | Check | Status | Conf | Sheet | Page |
| --- | --- | --- | --- | --- | --- |
| C-01 | Smoke alarm — all sleeping rooms and common areas | PASS | MEDIUM | p.2a1, p.2a2, p.2a3, p.2b | 13, 17, 18, 19 |
| C-02 | Architect stamp and signature — current | PASS | HIGH | p.2a1, p.2a2, p.2a3, p.2b, p.3e1, p.3a2, p.3b2, p.3e1-ADU | 13–24 |

## Structure

| ID | Check | Status | Conf | Sheet | Page |
| --- | --- | --- | --- | --- | --- |
| C-03 | Structural engineer stamp and signature | PASS | HIGH | S200, S300, S301, S302, S304, S400 | 26–32 |
| C-04 | Foundation type and structural design — adequacy | UNCLEAR | LOW | S200, S300, S301, S302, S304, S400 | 26–32 |

# Finding Details — FAIL & UNCLEAR

Observations and code references for all non-passing findings

A-01  Lot area — consistency across submittal documents  [FAIL]

Observation: Permit Application (page 4) states lot area = 7,330 sq ft. Site Development Data table (pages 5–6) states lot area = 7,730 sq ft. Two different values appear in the same submittal; difference is 400 sq ft.

Code: Austin LDC §25-1-22(A) — lot area is determined by survey; all permit documents must reflect the same verified survey measurement. Inconsistent lot area figures on a single submittal require correction and reconciliation to the recorded plat or current survey.

A-02  Use type designation — Two-Unit Residential vs. SFR+ADU  [FAIL]

Observation: The submittal title blocks describe the project as 'House w/ Habitable Attic' (architectural sheets) without explicitly designating the use type as Two-Unit Residential. Under HOME Phase I, two dwelling units on a single SF-3 lot constitute Two-Unit Residential use. The former Accessory Apartment use (§25-2-901) was repealed. Subchapter F GFA forms reference only the house, not the combined two-unit use. Unit count on permit app correctly shows 2 units.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773 — Duplex, Two-Unit, and Three-Unit Residential uses now govern two-unit development on SF-zoned lots; §25-2-901 (Accessory Apartments) repealed by same ordinance. Submittal must explicitly designate use type as Two-Unit Residential and apply §25-2-773 development standards.

A-03  HOME Phase I ordinance number — cited in applicable codes list  [UNCLEAR]

Observation: Plans cite 2021 IRC in plan notes (IRC R314, R315). The applicable codes list on the cover/administrative sheets does not clearly cite the HOME Phase I ordinance number (Ord. 20231207-001) or the governing LDC section (§25-2-773) in the visible reduced-scale review. At full-size sheet the citation may be present.

Code: Austin Administrative Code — permit submittals must list all applicable codes and local ordinances governing the project, including HOME amendment ordinance numbers. HOME Phase I, Ord. 20231207-001 must be cited as the governing standard for Two-Unit Residential use.

A-06  Front yard impervious cover — Phase I maximum 40%  [UNCLEAR]

Observation: Phase I §25-2-773(B) limits impervious cover in the front or street yard to 40% of that yard area. The site plan (p.1a, p.1d) shows a driveway and paved area in the front yard, but the percentage of the front yard that is impervious is not explicitly called out at the scale reviewed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(B) — maximum front yard impervious cover 40%. Front yard square footage and impervious area within the front yard must be calculated and shown on plans.

A-07  Front setback — house (primary building)  [UNCLEAR]

Observation: Phase I §25-2-773(B) requires minimum 15 ft front yard setback. The site plan and setback tent diagrams (pages 9, 21) show the house setback from the Euclid Ave (north) property line, but exact dimensioned setback values are not clearly legible at the scale reviewed. The setback appears to be at or near 15 ft.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(B) — minimum front yard setback 15 ft for Two-Unit Residential. Setback must be clearly dimensioned on site plan and verified against surveyed property line.

A-08  Rear setback — ADU (adjacent to alley)  [UNCLEAR]

Observation: Phase I §25-2-773(B) allows a rear setback of 5 ft when the rear lot line is adjacent to an alley or a lot permitted for multifamily or less restrictive use. The site plan shows an alley along the south property line. The ADU is the rearmost structure. The exact rear setback dimension from the south lot line is not legible at scale reviewed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(B) — minimum rear yard setback is base district minimum OR 5 ft if adjacent to alley. SF-3 base district rear minimum is 10 ft; alley adjacency reduces this to 5 ft. Rear setback must be dimensioned on site plan and verified.

A-09  Side setbacks — both buildings  [UNCLEAR]

Observation: The ADU ground floor plan (p.2b, page 19) notes '5' STREET SIDE SETBACK' and '20' FRONT STREET-SIDE SETBACK' at the lot boundaries. If this is a standard interior lot, SF-3 requires 5 ft interior side setbacks; if corner lot, Phase I requires greater of 5 ft from property line or 10 ft from curb (Level 1) or 10 ft from property line (Level 2–4) for the street-side yard. The lot's street-side condition is not definitively confirmed from the plans reviewed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(B) — corner lot: minimum street-side setback is greater of 5 ft from PL or 10 ft from curb (Level 1 street); or 10 ft from PL (Level 2–4 street). Interior side: Austin LDC §25-2-492, SF-3 base district, 5 ft minimum. All side setbacks must be dimensioned on site plan.

A-10  Front porch setback — house  [UNCLEAR]

Observation: The house north (street-facing) elevation shows a covered front porch projecting toward Euclid Ave. Phase I §25-2-773(C) requires that a porch projecting into the front yard be no closer than 15 ft from the front lot line, and that the porch roof/overhang be no closer than 13 ft. The exact porch location relative to the front lot line is not confirmed at scale reviewed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(C) — porch projecting into front yard must be ≥15 ft from front lot line; porch roof/overhang must be ≥13 ft from front lot line. These dimensions must be shown on site plan or elevation.

A-11  Parking — front/street yard spaces  [UNCLEAR]

Observation: Phase I §25-2-773(B) limits parking in the front or street yard to a maximum of 4 spaces (combined for corner lots). The site plan shows driveway access and parking area configuration but exact parking space count in the front/street yard is not confirmed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(B) — maximum 4 parking spaces in front or street yard combined. Parking count and location must be shown on site plan.

B-01  House per-unit GFA — Subchapter F FAR limit (Two-Unit Residential)  [UNCLEAR]

Observation: The Subchapter F GFA form (pages 7–8) shows a FAR of approximately 39.86% for the house (Unit 1). Using the lower lot area of 7,330 sq ft: house GFA ≈ 7,330 × 0.3986 ≈ 2,922 sq ft. Phase I §25-2-773(E) per-unit maximum for Two-Unit Residential is greater of (0.4 × 7,330 = 2,932 sq ft) or 2,300 sq ft = 2,932 sq ft. Margin: ~10 sq ft. The GFA calculation must be verified at full scale; a 10 sq ft margin is within the rounding tolerance of reduced-sheet review. If the correct lot area is 7,730 sq ft, the per-unit max becomes 3,092 sq ft and the house easily complies.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(E) — within Subchapter F compatibility area, Two-Unit Residential: per-unit maximum GFA is greater of 0.4 FAR or 2,300 sq ft. House GFA ~2,922 sq ft vs. max ~2,932 sq ft using 7,330 sq ft lot area. Exact GFA must be confirmed at full scale and lot area discrepancy (finding A-01) resolved first.

B-02  ADU per-unit GFA — Subchapter F FAR limit not shown  [FAIL]

Observation: The Subchapter F GFA form (pages 7–8) covers only Unit 1 (House w/ Habitable Attic). No Subchapter F GFA calculation is provided for Unit 2 (the ADU). Under Two-Unit Residential use, §25-2-773(E) sets a per-unit FAR maximum for each unit. The ADU's GFA must be calculated and submitted. From the ADU floor plans (page 19), the ADU includes ground-floor living area (with attached garage) and upper-floor bedroom/bath; exact GFA is not calculable without dimensioned plans at full scale.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(E) — within Subchapter F compatibility area, each unit in a Two-Unit Residential development must individually not exceed the greater of 0.4 FAR or 2,300 sq ft. Austin LDC Subchapter F §3.3 — GFA calculation methodology applies to all structures on a Subchapter F lot. ADU GFA calculation must be submitted.

B-03  Combined site GFA — Subchapter F site FAR limit (Two-Unit Residential)  [FAIL]

Observation: The combined site GFA (House + ADU) has not been calculated and presented in a single Subchapter F summary. Phase I §25-2-773(E) sets a site-level maximum: greater of 0.55 FAR or 3,200 sq ft for Two-Unit Residential. Using 7,330 sq ft lot: site max = greater of (0.55 × 7,330 = 4,031 sq ft) or 3,200 sq ft = 4,031 sq ft. House GFA alone is ~2,922 sq ft. ADU GFA is unknown (no calculation submitted). Without ADU GFA, site-level FAR compliance cannot be confirmed.

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(E) — within Subchapter F compatibility area, Two-Unit Residential: site maximum GFA is greater of 0.55 FAR or 3,200 sq ft. Both structures' GFA must be combined and compared to this site limit. A combined Subchapter F GFA exhibit covering both units must be submitted.

B-07  Street-facing entrance — ADU (Unit 2)  [FAIL]

Observation: The ADU north elevation (p.3b2, page 24) — the elevation facing toward Euclid Avenue (the primary street) — shows windows but no entry door on the north face. The ADU's primary entry door appears on the east elevation (page 23), accessed via exterior stairs on the east side of the building. Phase I requires a street-facing entrance for each building. The east elevation faces an interior side yard, not the primary street (Euclid Ave to the north).

Code: HOME Phase I, Ord. 20231207-001, §25-2-773(C) — minimum 1 street-facing entrance required. The street for this lot is Euclid Ave (north). The ADU's north elevation shows no entry door — a street-facing entry on the north elevation of the ADU must be provided, or the submittal must demonstrate an alternative compliant street-facing entrance.

C-04  Foundation type and structural design — adequacy  [UNCLEAR]

Observation: Foundation detail sheet (S200) shows reinforced concrete footings and grade beams with embedded plate connections. Structural framing plans (S300, S301, S302, S304) show LVL beams, floor trusses at 24" O.C., shear walls with CS-WSP panels, and wall bracing per seismic category A / wind 115 mph. Structural engineer stamp is present. Adequacy of the structural design for actual site soil conditions requires field-level engineering review.

Code: [REVIEWER: ___] — Structural adequacy of foundation, framing, and lateral bracing system requires review by City structural plan checker. Engineering stamp confirms design responsibility; field conditions to be confirmed at inspection. Reference: 2021 IBC Chapter 16 (structural loads), Chapter 18 (soils and foundations), Chapter 23 (wood frame construction).

# HOME Phase I/II Code Verification

Source: home_compliance.json  ·  Phase 3A — HOME Phase I/II code verification

| Reference files | home-phase-1.md (Ord. 20231207-001); home-phase-2.md (Ord. 20240516-006) |
| --- | --- |
| Findings reviewed | 14 |
| Verified | 14 |
| Corrected | 0 |
| Dropped | 0 |
| Phase II applicable | No — Lot is 7,330 sq ft — exceeds 5,750 sq ft threshold. Phase II (§25-2-779) does not apply. No findings incorrectly cite Phase II. |
| Summary | All 14 FAIL/UNCLEAR findings have accurate code citations confirmed against HOME Phase I reference. No findings dropped. No Phase II citations incorrectly applied. All five FAIL items and all UNCLEAR items carry valid, verifiable code bases. |

## Per-Finding Verdicts

| ID | Verified | Notes |
| --- | --- | --- |
| A-01 | ✓ Yes | §25-1-22(A) is a valid LDC cite for lot measurement consistency. Governing phase LDC is correct. |
| A-02 | ✓ Yes | Ord. 20231207-001 §25-2-773 and §25-2-901 repeal confirmed verbatim in Phase I reference. |
| A-03 | ✓ Yes | HOME Phase I Ord. 20231207-001 ordinance number and §25-2-773 confirmed. PHASE_I designation correct. |
| A-06 | ✓ Yes | §25-2-773(B) 40% front yard impervious cover limit confirmed verbatim in Phase I coverage table. |
| A-07 | ✓ Yes | §25-2-773(B) 15 ft minimum front yard setback confirmed in Phase I setback table. |
| A-08 | ✓ Yes | §25-2-773(B) rear setback 5 ft alley exception confirmed verbatim in Phase I reference. |
| A-09 | ✓ Yes | §25-2-773(B) corner lot street-side setbacks confirmed: Level 1 = greater of 5 ft from PL or 10 ft from curb; Level 2-4 = 10 ft from PL. |
| A-10 | ✓ Yes | §25-2-773(C) porch ≥15 ft from front lot line and overhang ≥13 ft confirmed verbatim in Phase I design standards. |
| A-11 | ✓ Yes | §25-2-773(B) maximum 4 parking spaces in front/street yard confirmed in Phase I parking section. |
| B-01 | ✓ Yes | §25-2-773(E) per-unit max greater of 0.4 FAR or 2,300 sq ft confirmed in Phase I FAR table. Two-Unit Residential category correct. |
| B-02 | ✓ Yes | §25-2-773(E) per-unit GFA maximum confirmed. Subchapter F §3.3 methodology reference is a valid supplementary LDC cite. |
| B-03 | ✓ Yes | §25-2-773(E) site max greater of 0.55 FAR or 3,200 sq ft for Two-Unit Residential confirmed verbatim in Phase I FAR table. |
| B-07 | ✓ Yes | §25-2-773(C) minimum 1 street-facing entrance requirement confirmed in Phase I minimum entrances section. |
| C-04 | ✓ Yes | IBC Chapter 16/18/23 citations are correct for structural review. Not a HOME standard — reviewer blank appropriate. |

# Texas State Law Analysis

Source: state_compliance.json  ·  Phase 3B — Texas state law verification

| Reference files | texas-state-law.md (SB 15, SB 840, HB 24 — 89th Legislature 2025) |
| --- | --- |
| Lot area | 7330 sq ft |
| Is small lot (≤4,000 sf) | false — SB 15 small-lot provisions DO NOT apply |
| SB 15 in effect | Yes — effective 2025-09-01 |
| SB 840 applicable | No — SB 840 applies to zones permitting commercial/office/retail/warehouse uses. SF-3 is a residential base zone. Not applicable. |
| HB 24 applicable | No — HB 24 governs zoning-change legislative procedures — not a plan review development standard. |
| Preemption found | None — No Texas state law preemption is triggered on any finding. The lot is 7,330 sq ft — nearly double th... |
| Local provisions prohibited | None identified. |

## Per-Finding Preemption Analysis

| ID | Preemption | Notes |
| --- | --- | --- |
| A-01 | None | Lot area documentation accuracy — not addressed by any state statute. |
| A-02 | None | Use type designation — not addressed by state statute. |
| A-03 | None | Ordinance citation on cover sheet — not addressed by state statute. |
| A-06 | None | SB 15 §211.055(a)(4) caps open space requirement at 30% for small lots only (≤4,000 sf). This lot is 7,330 sf — small-lot provision does not apply. HOME Phase I 40% front yard impervious limit governs. |
| A-07 | None | SB 15 §211.055(a)(1) caps front setback at 15 ft for small lots only (≤4,000 sf). Not applicable here. Phase I 15 ft front setback governs and equals the state cap regardless. |
| A-08 | None | SB 15 §211.055(a)(1) caps rear setback at 10 ft for small lots only. Not applicable. Phase I 5 ft rear setback (with alley) is more permissive than the state floor — no conflict. |
| A-09 | None | SB 15 §211.055(a)(1) caps side setback at 5 ft for small lots only. Not applicable. Local 5 ft side setback equals state floor — no conflict even if applicable. |
| A-10 | None | Porch setbacks are not wall articulation requirements. SB 15 §211.055(a)(5) prohibits wall articulation requirements on small lots only. Not applicable here. |
| A-11 | None | SB 15 §211.055(a)(2) prohibits covered parking requirements for small lots. SB 15 §211.055(a)(3) caps parking at 1 per unit for small lots. Neither applies to this 7,330 sf lot. Phase I maximum 4 spaces in front yard is a cap, not a minimum — not preempted. |
| B-01 | None | SB 15 §211.055(a)(6) prohibits building bulk restrictions for small lots only. SB 840 FAR preemption applies to commercial zones only. Neither applies here. Phase I §25-2-773(E) Subchapter F FAR limits govern. |
| B-02 | None | Same as B-01. |
| B-03 | None | Same as B-01. |
| B-07 | None | Street-facing entrance requirement is not a wall articulation standard. SB 15 §211.055(a)(7) prohibits wall articulation requirements for small lots only. Not applicable. |
| C-04 | None | IBC structural standards — not addressed by SB 15, SB 840, or HB 24. |